# Product Requirements Specification

Honda Vezel Custom Android Head-Unit Launcher  •  PRD-001  •  Version 1.0

# 1. Purpose and product vision

Create a premium, offline-first, landscape Android home launcher for a Honda Vezel head unit. The reference dashboard uses a digital speedometer on the left, media and weather cards at the top, a prominent central Vezel visualization, and a bottom dock. The launcher is a replaceable home application, not a firmware replacement or Android Auto projection app.

# 2. Reference dashboard

Design reference supplied by the project owner. Recreate as an original implementation rather than copying third-party application assets.

# 3. Target platform and constraints

| Item | Baseline / verification |
| --- | --- |
| Head-unit identity | HONDA_W15 / alps 8227L_demo; AC8227L |
| OS compatibility | Device Info HW: Android 8.1 / API 27. Factory About page reports Android 15; prioritize API reading, validate by APK testing. |
| CPU / ABI | 4 × ARM Cortex-A7; armeabi-v7a, 32-bit |
| GPU | Mali-450 MP; OpenGL ES 2.0 |
| Display | 1280 × 720 px, landscape; 160 dpi; 60 Hz |
| Memory | 4 GiB reported; 220 MiB free at capture; re-test under idle conditions |
| Install | APK side-load via trusted USB source; no ADB required |
| OEM compatibility | Keep factory radio, phone, reverse camera, and steering control routes intact |

# 4. User journeys

Start vehicle → home dashboard appears promptly after head-unit wake/reboot.

Tap Navigation → preferred installed map application opens.

Tap Music → chosen player opens; compatible playing track appears on dashboard.

Tap Apps → installed launchable applications appear in a categorized grid.

Park vehicle → Settings → change Vezel paint color; choice persists after restart.

When GPS or internet is unavailable, show graceful placeholders instead of stale data.

Return to factory home through a prominent recovery action until pilot sign-off.

# 5. Feature requirements

| ID / priority | Requirement | Acceptance criteria |
| --- | --- | --- |
| FR-01 / P0 | Home and navigation dock | 1280×720 UI with large touch targets; all dock shortcuts open configured apps. |
| FR-02 / P0 | Vehicle hero | Clearly recognizable Vezel model graphic centered with shadow and glow; no frame drops in idle. |
| FR-03 / P0 | Paint color | At least 6 swatches; same vehicle silhouette/perspective; persists after app restart. |
| FR-04 / P0 | Clock / date | Use system locale and time; updates reliably across wake and time changes. |
| FR-05 / P0 | Apps drawer | Show user-launchable apps; handle missing/uninstalled app targets. |
| FR-06 / P0 | GPS speed | Shows km/h from valid location fix; distinguish unavailable GPS from 0 km/h. |
| FR-07 / P0 | OEM shortcuts | Configurable link to existing radio/Bluetooth/phone apps; no direct MCU manipulation. |
| FR-08 / P0 | Safe recovery | Always offer path to stock launcher/system settings before default-home switch. |
| FR-09 / P1 | Now playing | Track title/art and controls when compatible accessible media session exists; fallback tile otherwise. |
| FR-10 / P1 | Weather | Consent-based location and online service; hide widget gracefully offline. |
| FR-11 / P1 | Theme | Dark/day/auto preference and saved accent palette; accessible contrast. |
| FR-12 / P2 | Car animations | Subtle lights, appearance and tilt animation; completely optional. |
| FR-13 / P2 | True 3D | Evaluate only after benchmarks and model licensing; not required for initial release. |
| FR-14 / P2 | Vehicle telemetry | OBD-II or OEM integration only with supported hardware and explicit validation. |

# 6. Screen specification

| Screen | Content / interaction |
| --- | --- |
| Dashboard | Header greeting/date/time; speedometer left; media center top; weather right top; car hero middle; dock bottom. |
| Apps | Installed app grid, search/filter, configurable pinned apps. |
| Settings | Car model/render color, theme, startup behavior, preferred apps, diagnostics, factory-home shortcut. |
| Vehicle appearance | Paint swatches and preview; optionally choose viewing angle from pre-rendered set. |
| Diagnostics | OS/API, memory, GPU, app versions, permissions status, safe-reset action. |

# 7. Non-functional requirements

Target smooth 30 fps on dashboard, subject to hardware measurement; avoid persistent high GPU load.

Prefer < 150 MiB launcher process proportional-set size (PSS) as an initial benchmark target, not a guaranteed limit.

Offline-first home view; no blocking network request during launcher startup.

All key primary controls accessible within one tap, with approximate 48 dp minimum touch targets.

No system/MCU flashing, root requirements, or risky changes to car safety systems.

No video playback promoted on the home screen while driving; setup actions intended for parked use.

Avoid storing precise trip history by default; request minimal Android permissions.

# 8. Scope and release criteria

MVP included: home screen, car renders/color selection, app launcher/dock, clock, GPS speed, persistent preferences, OEM shortcuts, safe rollback. P1 release: weather, media session UI, themes. Explicitly excluded from MVP: full 3D model, automatic doors/headlamps from CANBUS, reverse-camera takeover, custom radio implementation, firmware modifications.

Release gate: test cold boot, warm wake, 30-minute dashboard soak, map/audio switching, GPS unavailable, Wi-Fi unavailable, camera engagement with the stock app, stock launcher recovery and home-role behavior. No default launcher setting before recovery tests pass.

# 9. Risks and unresolved items

Exact Honda Vezel generation/model year and allowed vehicle model asset license must be confirmed.

Mali-450 performance, current memory pressure, and true head-unit home-app restrictions need physical-device tests.

Factory app package names and media-session behavior are device-specific; capture manually on the unit.

Weather API/provider, consent UX and connectivity availability are TBD.

Android 8.1 app permissions and OEM launch behavior need testing even when Android 15 is displayed in factory settings.